Notes to cabinet plans and elevations

7 Akarana Road, Chirnside Park

All cabinets have 	20mm bench overhang

Door fronts and panels to have 65mm V groove vertical unless noted otherwise

Brushed aluminium kickers

View 1		6 equal size drawers, J pull to top

16mm filler at each side

View 2		900 corner unit with lazy susan	Titus 607.8025.080.090

1200 double oven space with 32mm divider panel between ovens

40mm filler above, 128 mm drawer under

Drawer unit, 120 top and 2 equal under, J pull to top

16mm filler at RH end

Overhead	16mm filler at each side

Titus Skyrise opening system to lift up doors

Range hood cabinet to suit Kleenmaid RHUC92

Push catches to doors

View 3		16mm filler behind cupboard in view 5

450 pull out bin	2/37litre	Titus 607.8605.374.174

Drawer unit 3 equal size, J pull to top

800 sink cupboard, centre shelf with 3 vertical divider panels under

600 integrated dishwasher, allow kicker to run through this space

900 corner unit as per view 2

View 4		2 drawer units, 120 top drawer and 3 equal under, J pull to top

16mm filler to side of cupboard in view 5

View 5		16mm filler to LH side

Cupboard with centre shelf, push catch

Fixed panel to the floor

View 6		200 wide fixed panel under 600 high cupboard, centre shelf

Titus Skyrise opening system to lift up door

16mm side panel (no V groove)

3 equal size drawers, J pull to top

Slide back pocket doors, J pull to bottom

Hettich KA5740 mechanism and CA9079526 accessory set

Internal shelf unit fixed shelves at 580, 880, 1180 above bench top 300 deep

16mm filler panel to RH side (no V groove)